Q1. Capture and analyze ARP packets using Wireshark. Inspect the ARP request and reply frames, and discuss the role of the sender's IP and MAC address in these packets.

Source : Laptop IP - 172.17.24.74

Ping to 172.17.24.71 system in LAN

Network Traffic in wireshark

ARP request packet :

ARP reply packet :

1. ARP Request Frame Structure

An ARP Request is broadcasted in the network to find the MAC address corresponding to a given IP address.

- Sender MAC Address: The MAC address of the device sending the request.

- Sender IP Address: The IP address of the sender requesting the MAC.

- Target MAC Address: Set to 00:00:00:00:00:00 (unknown).

- Target IP Address: The IP address whose MAC address is being requested.

- Destination MAC Address: FF:FF:FF:FF:FF:FF (broadcast).

#### 2. ARP Reply Frame Structure

An ARP Reply is a unicast response from the requested device providing its MAC address.

- Sender MAC Address: The MAC address of the device responding.

- Sender IP Address: The IP address of the device responding.

- Target MAC Address: The MAC address of the requesting device.

- Target IP Address: The IP address of the requesting device.

- Destination MAC Address: MAC address of the request sender (unicast).

### Role of Sender's IP and MAC Address

- In ARP Request:

  - The sender's IP address is used so that the recipient knows who is making the request.

  - The sender's MAC address is included so that the recipient can send the ARP Reply directly back.

- In ARP Reply:

  - The sender's IP and MAC address provide the requested mapping.

  - The requester's MAC address ensures the reply reaches the correct device.

Q2.Using Packet Tracer, simulate an ARP spoofing attack. Analyze the behavior of devices on the network when they receive a malicious ARP response.

PC-1: 192.168.1.10

PC-2: 192.168.1.20

PC-3 (Attacker): 192.168.1.30

Changed the MAC address of PC2 with MAC address of PC3

Ping from PC1 , PC2 can’t be reached as PC2 and PC3 has same MAC address

Q3.Manually configure static IPs on the client devices(like Pc or your mobile phone) and verify connectivity using ping.

Configure a PC with IP : 172.17.24.71 with 255.255.255.0

Q4.Use Wireshark to capture DHCP Discover, Offer, Request, and Acknowledge messages and explain the process.

Laptop IP : 192.168.104.159

Router IP : 192.168.104.131

The DORA process is a four-step mechanism used by the Dynamic Host Configuration Protocol (DHCP) to assign an IP address dynamically to a client.

#### 1. Discover (D)

- The client (new device) broadcasts a DHCPDISCOVER message to find available DHCP servers.

- Source IP: 0.0.0.0 (since the client has no IP yet).

- Destination IP: 255.255.255.255 (broadcast).

#### 2. Offer (O)

- A DHCP server responds with a DHCPOFFER, proposing an available IP address.

- The offer includes:

  - Available IP address.

  - Subnet mask, gateway, and lease duration.

  - Server's IP address.

#### 3. Request (R)

- The client responds with a DHCPREQUEST to accept the offered IP.

- The request is broadcasted, allowing other DHCP servers (if any) to know the client accepted this offer.

#### 4. Acknowledge (A)

- The DHCP server sends a DHCPACK message, confirming the lease.

- The client configures the assigned IP and network settings.

DORA Process in DHCP : Discover

Offer :

Request :

Acknowledgement :

Q5.Given an IP address range of 192.168.1.0/24, divide the network into 4 subnets.

Task: Manually calculate the new subnet mask and the range of valid IP addresses for each subnet.

Assign IP addresses from these subnets to devices in Cisco Packet Tracer and verify connectivity using ping between them.

IP Address Range: 192.168.1.0/24 – Class C

Default Subnet Mask: 255.255.255.0 for Class C

11111111. 11111111. 11111111.00000000

Total Available Hosts: 2^8 - 2 = 254 (since /24 means 8 bits for host IDs)

To divide 192.168.1.0/24 into 4 subnets, we need to borrow 2 bits from the host portion:

| Before Subnetting | After Subnetting |
| --- | --- |
| /24 → 255.255.255.0 | /26 → 255.255.255.192 |
| Hosts per subnet: 254 | Hosts per subnet: 2^6 - 2 = 62 111110 – 6 bits |

New Subnet
 11111111. 11111111. 11111111.11000000 – 255.255.255.192 or /26

New Subnets and Address Ranges

| Subnet | Network Address | First Usable IP | Last Usable IP | Broadcast Address |
| --- | --- | --- | --- | --- |
| Subnet 1 | 192.168.1.0/26 | 192.168.1.1 | 192.168.1.62 | 192.168.1.63 |
| Subnet 2 | 192.168.1.64/26 | 192.168.1.65 | 192.168.1.126 | 192.168.1.127 |
| Subnet 3 | 192.168.1.128/26 | 192.168.1.129 | 192.168.1.190 | 192.168.1.191 |
| Subnet 4 | 192.168.1.192/26 | 192.168.1.193 | 192.168.1.254 | 192.168.1.255 |

Q6.You are given three IP addresses: 10.1.1.1, 172.16.5.10, and 192.168.1.5.

Task: Identify the class of each IP address (Class A, B, or C). What is the default subnet mask for each class?

Provide the range of IP addresses for each class.

| Class | First Octet Range | Default Subnet Mask | IP Address Range |
| --- | --- | --- | --- |
| Class A | 1 - 126 | 255.0.0.0 (/8) | 1.0.0.0 - 126.255.255.255 |
| Class B | 128 - 191 | 255.255.0.0 (/16) | 128.0.0.0 - 191.255.255.255 |
| Class C | 192 - 223 | 255.255.255.0 (/24) | 192.0.0.0 - 223.255.255.255 |
| Class D (Multicast) | 224 - 239 | N/A | 224.0.0.0 - 239.255.255.255 |
| Class E (Experimental) | 240 - 255 | N/A | 240.0.0.0 - 255.255.255.255 |

10.1.1.1 → Class A (Default Subnet Mask: 255.0.0.0)

172.16.5.10 → Class B (Default Subnet Mask: 255.255.0.0)

192.168.1.5 → Class C (Default Subnet Mask: 255.255.255.0)

Q7.  In Cisco Packet Tracer, create a small network with multiple devices (e.g., 2 PCs and a router). Use private IP addresses (e.g., 192.168.1.x) on the PCs and configure the router to perform NAT to allow the PCs to access the internet.

Task: Test the NAT configuration by pinging an external IP address from the PCs and capture the traffic using Wireshark.

What is the source IP address before and after NAT?

1. Network Setup

Devices Required

- Router0 (LAN Router)

- Router1 (ISP Router)

- Switch0

- PC0 and PC1

Connections

- Router0 (Gig0/0) → Switch0

- PC0 & PC1 → Switch0

- Router0 (Gig0/1) → Router1 (Gig0/0) (WAN link)

2. IP Configuration

PC0 & PC1 (Static IPs)

- PC0: 192.168.1.10/24, Gateway: 192.168.1.1

- PC1: 192.168.1.20/24, Gateway: 192.168.1.1

interface Loopback0

ip address 8.8.8.8 255.255.255.255

exit

ip route 0.0.0.0 0.0.0.0 200.200.200.1

3. Configure NAT on Router0

access-list 1 permit 192.168.1.0 0.0.0.255

ip nat inside source list 1 interface GigabitEthernet0/1 overload

interface GigabitEthernet0/0

ip nat inside

exit

4. Testing Connectivity

Verify NAT Translations

show ip nat translations

| Protocol | Inside Local | Inside Global | Outside Local | Outside Global |
| --- | --- | --- | --- | --- |
| ICMP | 192.168.1.10:1 | 200.200.200.1:1 | 8.8.8.8:1 | 8.8.8.8:1 |

Ping from PC0 to External IP

ping 8.8.8.8

5. Capture and Analyze Traffic

- Start packet capture on Router0 (Gig0/1).

- Ping 8.8.8.8 from PC0.

- Analyze source IP change before and after NAT:

  - Before NAT: 192.168.1.10 → 8.8.8.8

  - After NAT: 200.200.200.1 → 8.8.8.8