1)Try Test-Connection and nslookup commands for below websites

www.google.com

www.facebook.com

www.amazon.com

www.github.com

www.cisco.com

2.Use Wireshark to capture and analyze DNS, TCP, UDP traffic and packet header, packet flow, options and flags

DNS: Resolves domain names to IPs.

TCP: Connection-oriented, reliable communication with flags and sequence numbers.

UDP: Connectionless, faster but less reliable.

DNS Traffic Analysis

#### Filter DNS Packets: dns

#### Key Fields to Analyze:

- Transaction ID: Unique identifier for each query.

- Flags: Identifies query (0x0000) or response (0x8000).

- Queries & Responses: Shows domain resolution process.

- Record Type: A (IPv4), AAAA (IPv6), CNAME, MX.

#### Example DNS Query Analysis:

- A client requests www.google.com → Wireshark shows a query to a DNS server.

- The DNS server responds with an IP address.

### TCP Traffic Analysis

#### Filter TCP Packets: tcp

#### Key Fields to Analyze:

- Source & Destination Ports: Identifies the service (80, 443, 22).

- Sequence & Acknowledgment Numbers: Tracks communication flow.

- TCP Flags:

  - SYN (0x02) - Connection initiation.

  - ACK (0x10) - Acknowledgment.

  - FIN (0x01) - Connection termination.

  - RST (0x04) - Reset connection.

#### Example TCP 3-Way Handshake Analysis:

- SYN: Client sends SYN to initiate connection.

- SYN-ACK: Server responds with SYN-ACK.

- ACK: Client sends final ACK, establishing a connection.

### UDP Traffic Analysis

#### Filter UDP Packets: udp

#### Key Fields to Analyze:

- Source & Destination Ports: Example - 53 (DNS), 67-68 (DHCP).

- Length: Size of the UDP payload.

- Checksum: Validates packet integrity.

#### Example UDP Analysis:

- DNS requests typically use UDP.

3.Explore traceroute/tracert for different websites eg:google.com and analyse the parameters in the output and explore different options for traceroute command

### Columns Explanation

- Hop Number: Numbered list showing each router in the path.

- IP Address / Hostname: IP or resolved domain name of the router.

- RTT (Round Trip Time) in ms: Three response times indicate how long it took for a packet to reach the router and return.

### Key Observations

- * * *: Indicates a timeout or a firewall blocking ICMP packets.

- Increasing latencies (ms values): Can indicate network congestion or distance.

- Private IPs (192.168.x.x / 10.x.x.x): Represent internal networks.

Use Cisco packet tracer for the below

Q4. Set up trunk ports between switches and try ping between different VLANS

Configuration in switch 0 (VLAN 10):

Configuration in switch 1 (VLAN 20):

Configuration for Trunk port between Switch 0 and switch 1 :

Ping from PC0 to PC1 (same VLAN):

Ping from PC0 to PC2 (different VLAN):

Q5. Change the native VLAN on a trunk port. Test for VLAN mismatches and troubleshoot.

Two switches 0 and 1 connected with Crossover cable

Native VLAN configuration in switch 0:

Native VLAN mismatch issue:

Fix the mismatch in switch 1 :

Q6 .Configure a management VLAN and assign an IP address for remote access.

Test SSH or Telnet access to the switch.

Switch Configuration

SSH test at PC:

Q7. You have a Cisco switch and a VoIP phone that needs to be placed in a voice VLAN (VLAN 20).

The data for the PC should remain in a separate VLAN (VLAN 10). Configure the switch port to support both voice and data traffic.

Creating VLANs

Enable QoS ( for Voice Traffic)